Exp 26: Transaction Level Modeling (TLM) using SystemC

Program:

// Code your testbench here.

// Uncomment the next line for SystemC modules.

// #include <systemc>

#include <systemc>

#include <tlm>

#include <tlm_utils/simple_initiator_socket.h>  #include <tlm_utils/simple_target_socket.h>  using namespace sc_core;  using namespace tlm;  using namespace std;

// Target  struct Target : sc_module {  tlm_utils::simple_target_socket<Target> socket;  SC_CTOR(Target) : socket("socket") {  socket.register_b_transport(this, &Target::b_transport);

}

void b_transport(tlm_generic_payload& trans, sc_time& delay) {  int* data = (int*) trans.get_data_ptr();  cout << "Target received: " << *data << endl;  delay += sc_time(10, SC_NS);

}

};

// Initiator  struct Initiator : sc_module {  tlm_utils::simple_initiator_socket<Initiator> socket;

SC_CTOR(Initiator) : socket("socket") {

SC_THREAD(thread_process);

}

void thread_process() {  tlm_generic_payload trans;  sc_time delay = SC_ZERO_TIME;  int data = 555;  trans.set_data_ptr(reinterpret_cast<unsigned char*>(&data));  trans.set_data_length(sizeof(data));  cout << "Initiator sending: " << data << endl;  socket->b_transport(trans, delay);  wait(delay);

}  };

int sc_main(int argc, char* argv[]) {

Initiator initiator("initiator");  Target target("target");  initiator.socket.bind(target.socket);  sc_start();  return 0;

}

Output: