Exp No: 14 EMBEDDED C PROGRAM TO DISPLAY “HELLO WORLD” MESSAGE 
Github : https://github.com/Jaswanth-Sai878/Embedded-Lab-Experiments/tree/main

Program:

#include <reg51.h> #include <stdio.h> void UART_Init(void)

{

SCON = 0x50;      // Mode 1, 8-bit UART, enable receiver

TMOD = 0x20;      // Timer1 Mode2 (8-bit auto reload)

TH1  = 0xFD;      // 9600 baud rate for 11.0592 MHz crystal

TR1  = 1;         // Start Timer1

TI   = 1;         // Set TI for first transmission

}

char putchar(char c)

{

SBUF = c;         // Load character into UART buffer     while(TI == 0);   // Wait for transmission complete

TI = 0;     return c;

}

void main(void)

{

UART_Init();     while(1)

{

printf("Hello World!\r\n");

}

}

Out put: